Recommendation

Award

Executive Summary

Hillside is fairly new facility. Today it consists of two 18-hole courses (Hill Course and Valley course), both built on typical Finnish agriculture and forestry areas. The latest opening was in 2011, when 9 holes on Valley Course were opened. The course is members (and their guests) only, the first one operated in this manner in Finland.

Official regulation during the development has set site to be built up to date requirements for the buildings and the management of the golf course. Being built on former farming and forestry area with no special natural or heritage values means that the limitation for operation are directly involved with water, nutrient, pesticide and energy usage and are based n national regulations. Clubhouse, as well as the maintenance buildings have been planned according to the new standards.

Running the operations include efficient resource utilisation, conservation of water usage and water quality and the critical evaluation of materials used and purchasing decisions made. Plans for sustainable development have been integrated in everyday work.

The approach to environmental management of the course has been typically Finnish, doing more than stating. Future plans confirm, that there is willingness to do more. The sites application report covers the pros and cons of the work done exceptionally well and the report provides a very good source for understanding the site, the operations and the management attitude towards sustainable golf.

Hill Side on rakennettu perinteisenlle maa- ja metsätalousalueelle. Kentän sisällä ei ole tunnistettu erityistä huolenpitoa vaativia luonto, maisema tai kulttuuriperintökohteita.  Uudehko teknologia yhdistettynä aktiiviseen ja tavoitehakuiseen kestävän kehityksen mukaiseen toimintaan varmistaa, että kentän toiminnoista ei aiheudu edes maatalouden harjoittamisen mittakaavaisia kuormituksia ympäristölle. Osana paikallista yhteisöä on mahdollista tuottaa myös yhdyskuntateknisiä palveluja ratkaisuilla, joihin muutoin ei haja-asutusalueilla olisi BATNEEC - ajattelun mukaisesti olisi mahdollisuutta  Kentän arjen toiminnoissa näkyy yhteinen sitoutuminen kestävän kehityksen ajatteluun.

Landscape & Ecosystems

Observations

The site is about 40 kilometers from Helsinki. The flora and fauna of the area have been studied and no species with need for special attention have been found. Who did the study and when? Is it the luontoselvitys 2009 pdf as this is the only one in the club’s report? They ecosystem is not endangered nor disturbed beyond the limits of original usage (farming and forestry) of the area.

Planning has identified the typical features of the landscape and ecosystems and maintenance work keeps to good level of balance.

The ecosystem is also taken in consideration through using only slowly reacting and diluting nutrient sources. Erosion is monitored and supportive structures have been implemented where needed.

A comprehensive set of studies and thorough monitoring of the state of of the environment have been carried out. The future follow up plan has been developed by an independent expert, and it has been approved by local authorities.

Continual Improvement

For future development there might be a point to explore the option for further reducing erosion and evaluate the dirt movements within water systems

Water

Observations

Water usage is based on closed system with reservoir, which gets the needed refilling from river Vanjoki. There are no visible problems with the water (i.e. no excessive foaming etc.). Water quality monitoring has been frequent and consistent and a new programme for monitoring has been developed by an independent specialist.

The sites ecosystem has several waterhazards, that are (or a going to be) part of the sites water reservoir. The run offs' (i.e. rainfall) are directed to these waterhazards and hence supporting that there will be no nutrients going to the surrounding areas.

The site has a local sewage system and it's functionality is monitored on very frequent and regular schedule. The needed upgrades (as for the suppliers solutions didn't function as they should have) are done and functional to meet the standards of released water.

Continual Improvement

For future development some potential action points:

- explore the option for evaluating the fine particle movements within water systems.

- - continuing the originated development of irrigation being based amount of water sprayed instead of minutes sprayed

- Reduce irrigation (it seems quite a lot I think) going forward to accommodate new fescue grass

Energy & Resources

Observations

The main energy supply is electricity, which is bought only as a product of renewable sources. The areas where electricity is used is increasing, as carts are being upgraded to be battery operated. Course maintenance (mostly mowing) is done with petrol/diesel operated machines. Nutrients, pesticides, oils and other materials are stored in a proper way.

Energy consumption is monitored, the cost is understood and the potential for future savings is seen. Original actions show, that major savings can be achieved. Energy consumption should though be understood to be linked in both the usage of the site (both golf and other activities) and annual climate condition (harsh winters require more heating etc.).

Awareness of limited resources and action plans accordingly are helping the course to mitigate resource usage, even though this might not be seen in direct cost reduction (as the resource unit prices are most likely to increase).

There has been a number of initiatives (i.e. automatization and distance management of electricity, using only slowly diluting nutrients) in order to cut down consumption.

Continual Improvement

For future development some potential action points:

- explore the option for using lubricants, diesel and petrol which are not oil based products (i.e. for petrol E85, which is alcohol based)

- continuing the originated work on monitoring and mitigating electricity usage

- undertake a new system to maintain a log for hazardous waste disposal (which would record all, regardless of how small amount)

Products & Supply Chains

Observations

Purchasing is done from local sources as much as possible. Products are chosen based on their environmental impact and their applicability to the course. Good storage systems ensure, that materials are not wasted.

The site management has set a policy, in order to ensure that in purchasing and products follow the sustainable management principles. There is an active follow up of new developments in products and equipment and mental readiness to adopt new methods according to BATNEEC-philosophy.

Waste management is done according to the local municipality's instructions and waste storage and handling are done properly.

Continual Improvement

For future development some potential action points:

- explore the option for using aside of total amount of usages relative measures, where usage would be monitored per hectare or per round (as applicable)

- - continuing sorting of disposed waste and educating the players to be more active on this too

- - undertake a new information strategy of the approach taken towards the local society and the players ("cashing" the good work done)

Look to reduce fertilizer applications going forward, especially on fairways and roughs. When the turf has matured and is stronger this would also result in less irrigation requirements and better disease tolerance. Consider transition to greater % of organic fertilisers

Environmental Quality

Observations

As a new site, the basic setting for environmental quality is there. The every day work done is above the basic setting and requirements. Monitoring is done frequently and logs are well maintained.

Greens are constructed up to date. Grass species are chosen according to what functions best with minimum treatment and chemical support. The N usage on greens (142kg/ha in 2011) is far less than is allowed for farmland usage.

As a result of active monitoring of water, nutrients, pesticides and mowing need help to both keep resource utilisation to minimum and keeps deceases to the minimum.

The course are managed in a manner which actually supports reducing the free nutrient from the surface water. This shows the effectivity and balance of chosen management practices.

Continual Improvement

For future development some potential action points:

- explore the option for making log keeping of frequent unit visits (i.e. sewage treatment facility, oil separation units) more easy

- continuing the training of personnel and players of the importance and impact of every day decision and choices made

Look to reduce pesticide use in roughs

People & Communities

Observations

The site is a living and active part of the local community. A village "clubhouse" is situated within the clubhouse area. The game and the business are promoted to the local people by providing facilities and training, even free of charge, to them.

Local birdwatchers and the course have worked together to in order to provide birdlife monitoring sites on the course.

Personnel is aware of sustainable golf (in its' wide meaning) and they receive appropriate training.

Communication of the environmental work and protective actions taken are communicated to the players and other course shareholders. The local people and municipalities are informed, but more information could be delivered and also future development plans could be opened as a promise of further development and for discussions.

The environmental work is everyones responsibility and right. All levels are active and support the work. Also employees do initiate action, which support sustainable management (i.e. in clubhouse decisions are done on employee level).

Continual Improvement

For future development some potential action points:

- explore the option for improving the training logs to include

- an alarm element, so that outdating competence test are renewed on time

- the training and competences people have acquired (specially seasoned personnel) somewhere else  - continuing the training of personnel with the element of "walking the talk" by getting the permanent staff to understand, that

A) their behavior is seen as role model from the seasoned people

B) when understating safety measures (i.e. not using safety shoes), they also face the risk of loosing insurance coverage, would an accident take place

- undertake a new "monthly walk", where on a rotation each member (atleast from the permanent staff) would review the clubhouse and maintenance facilities in order to identify corrective action which will keep the site in the top decile.

Documentation Reviewed

Environmental Policy: Yes

Environmental Management Plan: Yes

Action Plans and Project Proposals: Yes

Certification Report: Yes

Minutes of Meetings: Yes

Newsletters: Yes

External Surveys and Reports: Yes

Internal Reports: Yes

EIA Statement: Yes

Emergency Incident Plan: Yes

Environmental Data: Yes

Register of Accidents: Yes

Training Log: Yes

Awareness Raising Materials: Yes

Conclusion

The site has no fancy history, nor any special species. Hence there are no special actions or stories to be revealed. The management has seen sustainable golf as an asset both for the business and the local society. Environmental work is not a committee work, or an action from the management. The work naturally is lead by the management, but the attitude and everyday applying penetrates through the entire organisation.

The work done is far over the basic requirements and the development plans show, that there is the will and the way to go further. The people at Hill Side should be more proud of what they are doing and could be more active in telling of it! Like can be seen from the application.

It's a pleasure to promote this course for the certification.

Certification Highlights

Even the site has grown, the electricity usage from 2010 to 2011 was down almost 20 %!

Can you provide one more highlight please

And a quote for a possible press release would be great too. Something like a general comment about how good the club is on environmental and social matters, one of your favourite things about what they do, the energy reduction for example.